|  | Contacts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rocket name: |  | Intimidator 5 - flight IV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Owner: |  | JURRIAAN VAN DE BEEK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Club: |  |  |  |  |  |  |  |  |  |  | NERO |  |  |  |  |  |  |  |  |  |  |  |
|  | Mobile telephone: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  |  |  |  | 17/08/2015 |  |  |  |  |  |  |  |  |  |  |  |
|  | E-mail: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | General |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Type (Experimental/HP/Model): |  | EXPERIMENTAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Number of stages: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
|  |  |  | Total length: |  |  |  |  |  |  |  |  | Diameter: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Start weight: |  |  |  |  |  |  |  |  |  |  |  |
|  | Dimensions: |  | 259,4 |  |  |  |  | cm |  |  |  | 13,1 |  |  |  |  |  |  |  |  |  |  |  |  |  | cm |  |  |  |  |  |  |  | 23,700 |  |  |  |  |  |  |  | kg |  |  |  |
|  |  |  | Body: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fins: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Colours: |  | BLACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ORANGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Centre of Pressure: |  |  |  |  |  |  |  |  | Centre of Mass: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Static Margin: |  |  |  |  |  |  |  |  |  |  |  |
|  | Centre's (from nose-tip): |  | 201,0 |  |  |  |  | cm |  |  |  | 180,5 |  |  |  |  |  |  |  |  |  |  |  |  |  | cm |  |  |  |  |  |  |  | 1,56 |  |  |  |  |  |  |  | cal |  |  |  |
|  |  |  | With same motor: |  |  |  |  |  |  |  |  | With other motor: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Rocket flown before: |  | NO |  |  |  |  | # |  |  |  | YES |  |  |  |  |  |  |  |  |  |  |  |  |  | # |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Motor Specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Motor name: |  | KILO 6 grain KNSU PG - 90mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Multiple nozzle: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Manufacturer: |  | JURRIAAN |  |  |  |  |  |  |  |  | Validation date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 16 |  |  | / | 05 |  |  | / |  | 14 |  |  |  |
|  | Propellant composition: |  | KNDX 65/35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Casing id.: |  |  |  |  |  |  |  |  | Number of times used; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date last use; |  |  |  |  |  |  |  |  |  |  |  |
|  | Casing: |  | N.A. |  |  |  |  | # |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  | # |  |  |  |  |  |  |  | 25 |  | / | 04 |  |  | / |  | 15 |  |  |  |
|  |  |  | Propellant mass: |  |  |  |  |  |  |  |  | Total impulse: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Burning time: |  |  |  |  |  |  |  |  |  |  |  |
|  | Characteristics: |  | 4,560 |  |  |  |  | kg |  |  |  | 5700 |  |  |  |  |  |  |  |  |  |  |  |  |  | Ns |  |  |  |  |  |  |  | 1,5 |  |  |  |  |  |  |  | s |  |  |  |
|  |  |  | Type: |  |  |  |  |  |  |  |  | Necessary current: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | During: |  |  |  |  |  |  |  |  |  |  |  |
|  | Igniter: |  | FUSEHEAD |  |  |  |  |  |  |  |  | KNOWN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mA |  |  |  |  |  | KNOWN |  |  |  |  |  |  |  | s |  |  |  |
|  | Flight control |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Flight control device: |  | RDAS TINY S/n 6277. Apogee @ 25s; Main @ 500m. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Flight Characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Tower exit velocity: |  |  |  |  |  |  |  |  | Maximum velocity: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Impact range: |  |  |  |  |  |  |  |  |  |  |  |
|  | Characteristics: |  | 46 |  |  |  |  | m/s |  |  |  | 248 |  |  |  |  |  |  |  |  |  |  |  |  |  | m/s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | m |  |  |  |
|  |  |  | Apogeum altitude: |  |  |  |  |  |  |  |  | Apogeum time: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Touch down time: |  |  |  |  |  |  |  |  |  |  |  |
|  | Characteristics: |  | 2170 |  |  |  |  | m |  |  |  | 20,4 |  |  |  |  |  |  |  |  |  |  |  |  |  | s |  |  |  |  |  |  |  | 121 |  |  |  |  |  |  |  | s |  |  |  |
|  | Recovery systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Type (para/streamer): |  |  |  |  |  |  |  |  | Exit (Hatch/Seper.): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | How was decent velocity obtained: |  |  |  |  |  |  |  |  |  |  |  |
|  | Drogue parachute: |  | TUMBLE |  |  |  |  |  |  |  |  | SEPERATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Flight I 16-05-2014 |  |  |  |  |  |  |  |  |  |  |  |
|  | Main parachute: |  | TOROIDAL |  |  |  |  |  |  |  |  | SEPARATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Flight I 16-05-2014 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Eject time: |  |  |  | Eject altitude: |  |  |  |  |  | Baro used: |  |  |  |  | Descent velocity: |  |  |  |  |  |  |  |  |  |  |  |  |  | Descent duration: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Drogue parachute: |  | t=22 | s |  |  | 2100 |  | m |  |  |  |  |  |  |  |  |  | 27,8 |  |  |  |  |  | m/s |  |  |  |  |  |  |  |  |  | s |  |  |  |  |  |  |  |  |  |  |
|  | Main parachute: |  | t=88 | s |  |  | 300 |  | m |  |  |  |  |  |  |  |  |  | 6,8 |  |  |  |  |  | m/s |  |  |  |  |  |  |  |  |  | s |  |  |  |  |  |  |  |  |  |  |
|  | Tracking systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Y/N: |  | Frequency: |  |  |  |  |  |  |  |  |  |  | Signal (cont./interm.): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | GSM GPS TRACKER : |  |  |  | 900/1800 |  |  |  |  |  | MHz |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BEELINE GPS : |  |  |  | 433.950 |  |  |  |  |  | MHz |  |  |  |  | APRS – INTIM5 – every 5s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Audio beacon: |  |  |  | ??? |  |  |  |  |  | Hz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pyro systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Function: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Mechanism type: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Safe/Arm: |  |  |
|  | Pyro device 1 & 3: |  | SEPARATION CHARGE FIN CAN / AV BAY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pyro device 2 & 4: |  | SEPARATION CHARGE NOSE CONE / AV BAY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pyro device x: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | How is pyro disarmed: |  | SWITCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | Pay-load |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y/N: |  | Frequency: |  |  |  |  | Video/audio/data: |  |  |  |
|  | Downlink: |  |  |  |  |  |  | Hz |  |  |  |  |  |
|  | Data acquisition device: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Experiments: |  | Big Red Bee – Beeline GPS APRS tracker 100mW |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Mobius 1080p camera – looking down |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Key chain camera # 11 – facing upwards recording main parachute deployment |  |  |  |  |  |  |  |  |  |  |
|  | Date & signature |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Owner, for correct and complete data: |  |  |  |  |  | Safety Officer for inspection & approval: |  |  |  |  |  |  |
|  | Jurriaan van de Beek |  |  |  |  |  |  |  |  |  |  |  |  |

Please don’t forget to include:

- A drawing with the measurements of the rocket, for recalculation of the Centre of Pressure

- A thrust diagram (trust in Newton against time) of the used motor

- The RockSim file (if you have one)